# QSPI Flash Controller

Specification

## Overview

The QSPI Flash Controller is configured through an APB register interface. These registers allow software to configure flash commands, addresses, transfer modes, dummy cycles, data length, and monitor controller status.

# 1 Micro-architecture:

## 1.1 Overview:

### 1.2 APB:

#### 1.2.1 Indirect Command Engine Path

## 1.3 SPI Controller:

### 1.3.1 FSM:

# 2 Register Map

| Offset | Register Name | Width | Access | Description | Reset Value |
| --- | --- | --- | --- | --- | --- |
| 0x00 | CTRL | 4 bits | RW | Transaction control register | 0x0 |
| 0x04 | OPCODE | 8 bits | RW | Flash command opcode | 0x00 |
| 0x08 | ADDRESS | 24 bits | RW | Flash memory address | 0x000000 |
| 0x0C | MODEBYTE | 16 bits | RW | Optional mode/alternate bytes | 0x0000 |
| 0x10 | DUMMY | 15 bits | RW | Number of dummy cycles | 0x0000 |
| 0x14 | DATALEN | 6 bits | RW | Data transfer length | 0x00 |
| 0x18 | MODECFG | 2 bits | RW | Transfer mode configuration | 0x0 |
| 0x1C | TXDATA | 32 bits | WO | Transmit data register (TX FIFO write) | 0x00000000 |
| 0x20 | RXDATA | 32 bits | RO | Receive data register | 0x00000000 |
| 0x24 | STATUS | 3 bits | RO | Controller status flags | 0x0 |

## 2.1 CTRL Register (0x00)

Controls the type of flash operation and initiates transactions.

| Bit | Field | Description |
| --- | --- | --- |
| [0] | START | Starts a flash transaction |
| [1] | IS_READ | Enables read operation |
| [2] | IS_WRITE | Enables write/program operation |
| [3] | ERASE_EN | Enables erase operation |

## 2.2 OPCODE Register (0x04)

Stores the flash command opcode to be transmitted during the command phase.

| Bits | Field | Description |
| --- | --- | --- |
| [7:0] | OPCODE | Flash command opcode |

### 2.2.1 Supported Opcodes

| Opcode | Command |
| --- | --- |
| 0x06 | Write Enable (WREN) |
| 0x05 | Read Status Register (RDSR) |
| 0x9F | Read JEDEC ID (RDID) |
| 0x02 | Page Program (PP) |
| 0x20 | Sector Erase (SE) |
| 0x0B | Fast Read |
| 0xEB | Quad I/O Read |

## 2.3 ADDRESS Register (0x08)

Stores the target flash address.

| Bits | Field | Description |
| --- | --- | --- |
| [23:0] | ADDRESS | Flash memory address |

## 2.4 MODEBYTE Register (0x0C)

Stores optional mode or alternate bytes used by certain flash commands.

| Bits | Field | Description |
| --- | --- | --- |
| [15:0] | MODEBYTE | Mode/alternate byte value |

## 2.5 DUMMY Register (0x10)

Defines the number of dummy clock cycles inserted before data transfer begins.

| Bits | Field | Description |
| --- | --- | --- |
| [14:0] | DUMMY_CYCLES | Dummy cycle count |

Range

0 to 32767 cycles

## 2.6 DATALEN Register (0x14)

Specifies the amount of data transferred during the data phase.

| Bits | Field | Description |
| --- | --- | --- |
| [5:0] | DATA_LEN | Number of bytes to transfer |

Range

0 to 63 bytes

## 2.7 MODECFG Register (0x18)

Selects the transfer width used during communication with the flash.

| Bits | Field | Description |
| --- | --- | --- |
| [1:0] | MODE | Transfer mode selection |

### 2.7.1 Encoding

| Value | Mode |
| --- | --- |
| 2'b00 | Single SPI (1-bit) |
| 2'b01 | Dual SPI (2-bit) |
| 2'b10 | Quad SPI (4-bit) |
| 2'b11 | Reserved |

## 2.8 TXDATA Register (0x1C)

Write-only register used to push data into the transmit FIFO.

| Bits | Description |
| --- | --- |
| [31:0] | Data written to TX FIFO |

## 2.9 RXDATA Register (0x20)

Read-only register used to retrieve received data.

| Bits | Description |
| --- | --- |
| [31:0] | Data received from flash |

## 2.10 STATUS Register (0x24)

Provides the current status of the controller.

| Bit | Field | Description |
| --- | --- | --- |
| [0] | BUSY | Controller is executing a transaction |
| [1] | DONE | Transaction completed successfully |
| [2] | ERROR | Transaction terminated with an error |

### 2.10.1 Status Values

| ERROR | DONE | BUSY | Meaning |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Idle |
| 0 | 0 | 1 | Transaction in progress |
| 0 | 1 | 0 | Transaction completed |
| 1 | X | X | Error detected |

# 5. Design Operation

## 5.2 Command Flow

The controller supports a programmable transaction sequence:

- SPI Mode

- Dual SPI Mode

- Quad SPI Mode

This configuration is selected through the MODECFG register.

## 6.3 APB Register Bank

The APB Register Bank stores all transaction configuration parameters.

Software programs:

| Register | Function |
| --- | --- |
| CTRL_REG | Start transaction and operation type |
| OPCODE_REG | Flash command opcode |
| ADDR_REG | Flash address |
| MODEBYTE_REG | Optional mode byte |
| DUMMY_REG | Dummy cycle count |
| DATALEN_REG | Data transfer length |
| MODECFG_REG | Phase width configuration |
| TXDATA_REG | Write data |
| STATUS_REG | Controller status |

When start is asserted, the register contents are forwarded to the Indirect Command Engine.

## 6.4 Indirect Command Engine (ICE)

The ICE collects all programmed transaction parameters and creates a complete command descriptor.

The descriptor includes:

Opcode

Address

Mode Byte

Dummy Cycles

Data Length

Read/Write Information

Phase Enables

Transfer Width Configuration

The descriptor is written into the Command FIFO for execution.

## 6.5 Command FIFO

The Command FIFO stores pending flash transactions.

Functions:

- Decouples APB programming from command execution

- Buffers command requests

- Preserves transaction ordering

The QSPI Decoder  reads commands from the FIFO whenever the QSPI Controller becomes available.

## 6.7 QSPI Controller

The QSPI Controller performs the actual flash protocol execution.

The controller consists of:

QSPI Decoder

QSPI FSM

TX Shift Engine

RX Shift Engine

Clock Divider

IO Control Logic

The controller interprets the transaction descriptor and generates the required QSPI signaling.

## 6.9 Transfer Width Configuration

Each transaction phase supports programmable bus widths.

| Mode Value | Transfer Width |
| --- | --- |
| 2'b00 | Single SPI (1-bit) |
| 2'b01 | Dual SPI (2-bit) |
| 2'b10 | Quad SPI (4-bit) |
| 2'b11 | Reserved |

The following phases are independently configurable:

- Address Phase

- Mode Byte Phase

- Dummy Cycle Phase

- Data Phase

## 6.10 Data Transfers

### Read Operations

For read transactions:

Received data is made available through the APB interface.

### Write Operations

For write transactions:

Data is transmitted to the flash device during the Data Phase.

## 6.11 Status Reporting

Transaction status is reported through STATUS_REG.

| Bit | Signal | Description |
| --- | --- | --- |
| [0] | busy | Controller executing a transaction |
| [1] | done | Transaction completed |
| [2] | error | Transaction error detected |

## 6.12 Reset Behavior

All modules use an active-low asynchronous reset (PRESETn / RESETn).

Upon reset:

- Controller returns to Idle state

- All configuration registers are cleared

- FIFO interfaces are reset

- Status flags are cleared

- Pending transactions are discarded